Product Requirements Document

Component name

| Issued by | Role/Department | Date |
| --- | --- | --- |
| Nicolò Palmerini | Electronic Team Member | 21/10/2025 |

TABLE OF CONTENTS

GENERAL PRODUCT DESCRIPTION	3

DESIGN	3

INTERFACE	3

ELECTRICAL HARDWARE AND SENSORS	3

TEST DA FARE	3

BOM	3

REPORT	3

# GENERAL PRODUCT DESCRIPTION

| Product Description |  |
| --- | --- |
| Key Features/ Functionality | Non-contact rotary sensor based on Hall-Effect technology, designed for harsh motorsport environments; measures the angular position of mechanical components (e.g., throttle, pedals). Resolution: 12-bit @ 5 kHz. Configurable electrical angle (30° / 100° / 360° standard). Available in single or dual-channel versions for redundancy, with analog outputs of 0.5–4.5 V. Operating range: -40 °C to +150 °C, protection rating IP67, weight approximately 15 g. Overvoltage protection up to 40 V. |
| Software | No |
| Other components Compatibility, Ecosystem, etc. | Direct interface with ECU/MCU analog inputs (0–5 V); otherwise, a signal conversion is required. |
| Working conditions | pedal box, steering wheel. IP67 |
| Rules | APPS: T.4.2.1–T.4.2.6 Brake: T.4.3.1–T.4.3.5, T.3.4.2 Wiring: EV.6.6, EV.9.1–EV.9.4, T.9.2 Gen: EV.13.3, T.5.5 |
| Dimensions and Tolerances | 40x14 mm (C type on the datasheet) |
| Weight | 15g |
| Produzione | Aviorace |
| Connections |  |

# DESIGN

| Schematico | N/A |
| --- | --- |
| Renderings | N/A |

# INTERFACE

| User Interface | pedal box, steering wheel. |
| --- | --- |
| Communication protocol | Analog |
| Communication Input | N/A |
| Communication output | Analog voltage output of 0.5–4.5 V; ratiometric when supplied at 5 V. |
| Connector | Supplied with an unterminated cable. |
| Visual Interface (Screen size and type, LEDs) | None |
| External Software interface | N/A Diagnostic functions implemented at ECU level |

# ELECTRICAL HARDWARE AND SENSORS

| Block Diagram of Electrical Hardware |  |
| --- | --- |
| Supply Voltage | 5 V DC (ratiometric output, 10–90% of supply voltage). 6–30 V DC (absolute output, 0.5–4.5 V). |
| Supply Current | unspecifed |
| Input/Sensor Requirements | N/A |
| Output/Actuator Requirements | Analog voltage signal to ECU ADC (0–5 V range). Recommended ADC input impedance ≥100 kΩ. Suggested anti-aliasing filter: 1 kΩ + 100 nF (to be validated). Software-side requirements: signal linearization, dual-channel plausibility, 5–10 sample debounce filter. Maximum output current: 8 mA per channel (do not exceed). |
| OverVoltage/ OverCurrent / Short Circuit Protection on power supply | Reverse polarity protection integrated. Overvoltage protection up to 40 V DC. |
| OverVoltage/ OverCurrent / Short Circuit Protection on output | non-protected |

# BOM

| Bill of material |  |
| --- | --- |
| BOM |  |
| Financials |  |
| BOM Cost |  |